AN335

ALTERNATE PISTONS FOR V-4

In his article on rebuilding the SAAB V-4, Jack Ashcraft suggests using Ford truck pistons in the 1500cc version on the grounds that they're cheaper than oversize pistons from SAAB. I'd like to suggest an alternate approach that I used when I had my engine rebuilt: converted to 1700cc and stop buying premium gas. If your engine needs a rebore and a new crank anyway, which mine did, then the case for the conversion becomes pretty compelling.

Whether you have the early oddball open-deck V4 or the later closed deck variety, converting to 1700cc couldn't be much more straightforward: just use the appropriate crankshaft and pistons. As Jack points out, the connecting rods are the same. Phoning around gave me the impression that SAABers are less willing to part 1700 than with 1500 cranks, but the wrecking yards price is the same for either kind. How do you tell what kind of crank you have? Piece of cake-- 1,5 or 1,7 will be stamped into the web of the crank.

If you convert to 1700cc you can use pistons for the Capri 2600cc V-6 or for the 2000cc Pinto inline 4. The chart below gives vital statistics for stock-bore pistons of both kinds:

ÉÍÍÍÍÍÍÍÍÍÍÍÍÍÑÍÍÍÍÍÍÍÍÍÍÍÍÑÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÑÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍ»

º	TYPE	    ³   	TRW	  ³	Diameter	   ³	Compression        º

º		    ³   	P/N	  ³	mm/in.	   ³	distance           º

º 		    ³   		  ³			   ³	mm/in              º

ÇÄÄÄÄÄÄÄÄÄÄÄÄÄÅÄÄÄÄÄÄÄÄÄÄÄÄÅÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÅÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ¶

º 	Capri    ³   	L1255F ³	90.0/3.543   ³	39.27/1.546        º

º 	Pinto    ³   	L1252F ³	90.8/3.575   ³	40.51/1.595        º

ÇÄÄÄÄÄÄÄÄÄÄÄÄÄÁÄÄÄÄÄÄÄÄÄÄÄÄÁÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÁÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ¶

º	* compression distance (or open deck height) = distance    º º      from piston crown to centerline of wrist pin             º

º                                                               º

º                                                               º

ÈÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍ¼

Putting the Pinto pistons on your rods is no problem diameter of wrist pins is the same in either case. As the chart shows, thanks to the Pinto engines

's slightly larger bore, using Pinto pistons is equivalent to a .032" oversize in itself. The Pinto pistons' greater compression distance will raise compression and power very slightly, but you'll still be able to use regular gas. If you need to get by with just a ring and valve job some day, rings are easier to come by for the Pinto than the Capri pistons.

When you take your engine parts to the machine shop, don't make the mistake I made: I dropped off the 1700 crank, the new pistons, and the connecting rods with the old pistons still attached. When I phoned back a couple of days later, the machinist was holding off because he didn't know that the old pistons with their greater compression distance had originally gone on the shorter throw 1500 crank. Assuming that they'd gone on the 1700 crank, he concluded that I'd be getting a drastic drop in compression ratio. I could have saved the delay by explaining the situation.

Another tip on engine overhauls in general: check the machine shops work. The machine shop I used has a good reputation and did a good job on my engine for the most part, but someone installed the center camshaft bearing slightly crooked, partially blocking the oil holes. Some oil, but not enough, was getting to the upper end of the engine. I had to have the engine torn down again (scratch one set of gaskets) and the bearing rotated into the proper position. Glen Edinger informs me that some people handle this situation by drilling new holes in the bearing (just run a long bit through the oil holes) after installed.

Another tip from Gen Edinger: if you're using the Weber 34ICH carburetor, check your jetting, because you virtually get potluck out of the box. Of three 34ICH's Glen worked on recently, no two were jetted alike. Correct jetting is: main jet 195. air corrector jet 165.