AN368

LEAKING V-4 FUEL PUMP

If buying a BAP/GEON fuel pump for a V-4 you should be aware of a manufacturing problem they have. The pump will leak about three to four weeks after installation and it will appear that the input hose is the culprit.

Before installing the new fuel pump take the center screw off that exposes the plastic filter inside. Looking at the cover removed, there is a timer at the input side of the nipple. This needs to be filed down so it will not bottom out on the matting part. Also, the matting surface that contacts the plastic filter (also the gasket) needs to be filed.

Remember to rotate the angle of attack while getting this surface to clean up. It's very easy filing as it is a die casting. Before installing, ash or blow off the fillings. unfortunately SAAB no longer offers this part.