AN91R

CARB CONVERSION

I too had trouble with the Autolite carburetor on a '70 V-4 96 and solved it with a Solex transplant from a '68. no need to torch the throttle linkage though as it can easily be reversed by attaching a bracket.

Occasionally when hot, the engine would be very difficult to start, as if it were flooding over. Turned out the plastic float was filled with gas and had sunk. I've tried everywhere to get another but to no avail and finally plugged the hole with fiberglass resin. To find the hole and get the gas out, immerse the float for a couple minutes in 180 deg water to pressurize the float and rotate it. So far the resin hasn't leaked but I'd like to get another float if anyone knows of a source.